Logo and college name

Department

COMPUTER SCIENCE LEVEL 6

ICT/CU/CS/CR/01/6/A

UNDERSTAND COMPUTER ORGANIZATION AND ARCHITECTURE

END OF TERM EXAM – JULY –AUGUST 2025

Time: 2 hrs.

Instructions:

-Write your Name, admission Number, Course and Level in the provided answer sheets before starting the exam.

-All Answers should be written in the answer sheets provided.

-Answers should be written in English.

-The exam has two sections: A & B

-Attempt all questions.

ASSESSOR GUIDE

Section A: (40 marks)

- Define computer architecture. (2 marks)

Computer architecture is the design and organization of a computer's fundamental operational structure, including the hardware components, data formats, instruction sets, and system interconnections.

- List the three main components of the CPU (3 marks)

  Arithmetic Logic Unit (ALU)

  Control Unit (CU)

  Registers

- Outline the steps of the instruction execution cycle. (4 marks)

  Fetch – The CPU retrieves the instruction from memory.

  Decode – The instruction is interpreted by the control unit.

  Execute – The instruction is carried out (e.g., arithmetic operation).

  Store – The result is written back to memory or a register.

4. Convert the following:
a) 1101₂ to decimal (2 marks)
b) 25₁₀ to binary (2 marks)
c) 3F₁₆ to decimal (2 marks)
d) 77₈ to decimal (2 marks)

Answers:

a) 1101₂ to decimal = 13
b) 25₁₀ to binary = 11001₂
c) 3F₁₆ to decimal = 63
d) 77₈ to decimal = 63

- List four types of memory used in a computer system. (4 marks)

  Random Access Memory (RAM)

  Read-Only Memory (ROM)

  Cache Memory

  Virtual Memory
(Other valid types include: Flash Memory, Registers, Hard Disk, Solid State Drive,

- State two differences between primary and secondary memory (2 marks)

| Primary Memory | Secondary Memory |
| --- | --- |
| Temporary and volatile (e.g., RAM) | Permanent and non-volatile (e.g., HDD, SSD) |
| Faster access speed | Slower access speed |

6. Define the following logic gates with their symbols: (6 marks)
a) AND
b) OR
c) NOT

8. Give the truth table for the XOR gate (2 marks)

9. Highlight FOUR applications of logic gates in digital electronics. (4 marks)

10. a) Give the steps used to demorganize a Boolean expression. (3 marks)

b) Find the Boolean equation for the output X in the figure below : (2 marks)

X=(A+B)C

Section B (60 marks)

- a) Perform the following conversions as indicated: (10 marks)

- 0.610 to binary

- 1710  to binary

- 358  to decimal

- 24510  to octal

- 11010101.011012  to Octal

Answers:

i. 0.6₁₀ to binary (2 marks):
Multiply 0.6 by 2:
0.6 × 2 = 1.2 → 1 (integer part), carry 0.2
0.2 × 2 = 0.4 → 0, carry 0.4
0.4 × 2 = 0.8 → 0, carry 0.8
0.8 × 2 = 1.6 → 1, carry 0.6
(Repeats)

Thus,
0.6₁₀ ≈ 0.1001...₂ (recurring)

ii. 17₁₀ to binary (2 marks):
Divide 17 by 2:
17 ÷ 2 = 8 remainder 1
8 ÷ 2 = 4 remainder 0
4 ÷ 2 = 2 remainder 0
2 ÷ 2 = 1 remainder 0
1 ÷ 2 = 0 remainder 1

Reading remainders bottom to top:
17₁₀ = 10001₂

iii. 35₈ to decimal (2 marks):
Expand using powers of 8:
= (3 × 8¹) + (5 × 8⁰)
= (3 × 8) + (5 × 1)
= 24 + 5
= 29₁₀

iv. 245₁₀ to octal (2 marks):
Divide by 8:
245 ÷ 8 = 30 remainder 5
30 ÷ 8 = 3 remainder 6
3 ÷ 8 = 0 remainder 3

Reading remainders bottom to top:
245₁₀ = 365₈

v. 11010101.01101₂ to Octal (2 marks):
Group in threes from decimal point:

Left side (integer part):
011 010 101 → group 011 010 101

Right side (fractional part):
011 010 → group: (011)(010)

Convert each group:
011 – 3

010 – 2

101 – 5

Fractional part:

011 – 3

010 - 2

Thus,
11010101.01101₂ = 325.32₈

- Perform the following binary operations:

- Add 1010, 1001 and 1101 (3 marks)

- Multiply 1011 by 1010      (3 marks)

Answers:

i) 1010 + 1001 + 1101 = 100000 (add the first two, then to the result add the last one)

ii) 1101110 (carried out just like in base 10)

- Explain two advantages of using number systems (4 marks)

12 a) Perform the following number systems operations:

- Multiply 357 by 23 using octal (3 marks)

- 7438  - 5648   (3 marks)

- ABC16  × 2316   (3 marks)

- ADD16   +   DAD16  (3 marks)

Answers:

i) 357 x 23  = 106758  (use long multiplication method)

remember when we have 7x3 = 21 in octal written as 2x8 + 5 => 5 is written as the result carry 2 , every value  greater than 7 is written in form of  C0 x 8 + S0  where C0  is carry and S0  the sum/product or what goes to answer part.

ii) 743 – 564  =  1578  (remember when we borrow, we borrow 8 e.g 3-4 is not possible, borrowing from 4, we remain with 3 but we take 8 to column having 3, so we will have 3+8=11, now we take 11-4 = 7) and so on

iii) ABC x 23 = 177B416

iv) ADD + DAD = 188A16

b) Outline the memory hierarchy in computer systems. Clearly describe the role and speed of each level. Include a labeled diagram. (8 marks)

- Registers

  - Role: Smallest, fastest memory located inside the CPU. Stores immediate values for execution.

  - Speed: Fastest

  - Size: Very small (bytes)

- Cache Memory

  - Role: Stores frequently used instructions/data to reduce access time to main memory.

  - Speed: Very fast, slower than registers

  - Levels: L1 (fastest), L2, L3

  - Size: Small (KB–MB)

- Main Memory (RAM)

  - Role: Holds currently running programs and data used by CPU.

  - Speed: Moderate

  - Size: Medium (GBs)

- Secondary Storage

  - Role: Stores data and programs permanently (e.g., HDD, SSD).

  - Speed: Slow

  - Size: Large (GBs–TBs)

- Tertiary/Backup Storage

  - Role: Used for backups and archiving (e.g., magnetic tapes, cloud storage).

  - Speed: Slowest

  - Size: Very large

13. a) Discuss five major differences between RISC and CISC architectures. (10 marks)

| Feature | RISC (Reduced Instruction Set Computer) | CISC (Complex Instruction Set Computer) |
| --- | --- | --- |
| 1. Instruction Set | Uses a small, simple set of instructions. | Uses a large, complex set of instructions. |
| 2. Execution Time | Most instructions execute in a single clock cycle (faster). | Instructions may take multiple cycles due to complexity (slower). |
| 3. Instruction Format | Fixed-length instructions, simplifying decoding. | Variable-length instructions, requiring more complex decoding. |
| 4. Memory Usage | More instructions needed, so larger program size. | Fewer instructions due to complex operations, reducing code size. |
| 5. Hardware Complexity | Simplified control unit, making it easier and cheaper to design. | Complex control unit to handle many instruction types. |

b) You are designing a basic digital security system that opens a door only when two conditions are met:

- A correct password is entered.

- A fingerprint is verified.

a) Draw a logic gate circuit using appropriate gates to represent this system. (4 marks)
b) Describe how the system would behave if only one condition is met. (2 marks)
c) Identify one logic gate that could be used to represent this decision-making process and explain why. (4 marks)

Answers:

a) This system requires both conditions to be true (Password AND Fingerprint). Therefore, the logic gate used is an AND gate.

Door Opens

Finger print

password

b) If only one of the two conditions is met (either password is correct or fingerprint is verified), the AND gate output will be 0, so the door will remain closed.

c)

  Logic Gate: AND Gate

  Why:
The AND gate outputs true (1) only when both inputs are true. This perfectly models the requirement that both the correct password and fingerprint must be provided for the door to open.

--END OF EXAM PAPER---