KENYA JUNIOR SCHOOL EDUCATION ASSESSMENT

GRADE 7: 712/2: PRETECHNICAL STUDIES

CODE: 008   YEAR: 2025

MARKING SCHEME

General Guidance for Teachers (Non-Confidential)

Project Overview: This project aims to assess learners' practical skills in design, safety, digital literacy, and basic business understanding through the construction of a sieve for a porridge vendor. It encourages teamwork and the use of locally available resources.

Key Learning Outcomes:

Pre-Technical Skills: Sketching, measuring, cutting, assembling, material selection, safety awareness.

Computer Studies: Digital documentation (photography/videography), portfolio creation, ICT tool identification.

Business Studies: Entrepreneurial qualities, economic resources, make-or-buy decisions.

### Teacher's General Guide

1. Preparation and Planning (Before the Project Month)

- Review the Project Brief: Thoroughly understand all tasks and expectations outlined in the learner paper.

- Resource Identification:

  - Locally Available Materials: Brainstorm and list potential materials for a sieve (e.g., wood offcuts, plastic containers, old metal mesh/sieve material, strong fabric/netting, wire, nails, screws, glue). Emphasize resourcefulness.

  - Tools: Identify essential hand tools required (e.g., hacksaw, tenon saw, measuring tape/ruler, marking tools, hammer/screwdriver, scissors/snips, sandpaper).

  - ICT Equipment: Ensure access to smartphones/cameras for photo/video, computers/laptops, and basic photo/video editing software or presentation tools (e.g., Google Photos, PowerPoint, simple video editors, Google Drive for portfolio).

- Workshop/Learning Space Preparation:

  - Ensure a safe working environment.

  - Check tools are in good working condition.

  - Discuss safety protocols with learners beforehand.

- Group Formation: Form groups of 4-5 learners. Encourage diverse skill sets within groups.

- Initial Discussion with Learners:

  - Introduce the project scenario and tasks.

  - Clarify expectations regarding teamwork, individual contributions, and deadlines.

  - Emphasize safety and the use of locally available materials.

  - Explain the concept of an "electronic portfolio."

2. Project Implementation - Procedures (During the Project Month)

- Week 1: Planning and Design (Pre-Technical & Business)

  - Task 1 (Q1): Sketching: Guide groups to brainstorm sieve designs. Emphasize freehand pictorial sketching, showing important parts (frame, mesh, handle/support) and dimensions. Discuss how to represent different materials.

  - Task 1 (Q2): Safety: Facilitate a discussion on safety precautions when using cutting tools. Ensure learners understand the 'why' behind each precaution.

  - Task 1 (Q3): Drawing Lines: Review different types of lines in technical drawing, specifically hidden lines.

  - Task 1 (Q4): Marking & Measuring: Discuss the importance of accuracy in marking and measuring.

  - Task 3 (Q7): Entrepreneurial Qualities: Discuss the characteristics of an entrepreneur and how they apply to the porridge vendor.

  - Task 3 (Q8): Economic Resources: Guide learners to identify and classify resources.

- Week 2-3: Construction and Documentation (Pre-Technical & Computer Studies)

  - Practical Construction:

    - Oversee the material selection and collection by groups.

    - Guide learners through the cutting, shaping, and joining processes.

    - Emphasize adherence to safety precautions identified earlier.

    - Encourage problem-solving and collaboration within groups.

  - Task 2 (Q5): Photo/Video Documentation: Instruct learners to capture clear photos or short videos of each significant step:

    - Initial sketching and design

    - Measuring and marking materials

    - Cutting processes (with safety gear visible if possible)

    - Assembly of parts

    - The final assembled sieve

    - Teacher's role: Check for documentation progress regularly.

  - Task 2 (Q6): Electronic Portfolio: Guide groups on how to create a simple digital portfolio.

    - Suggest using cloud-based platforms (Google Drive, OneDrive) or presentation software (PowerPoint, Google Slides) where photos/videos can be embedded.

    - Emphasize naming the portfolio clearly (e.g., Group Simba Sieve Project).

    - Discuss common ICT tools for this (e.g., camera/phone, computer, specific software like Google Photos, PowerPoint).

- Week 4: Finalization and Business Analysis (Business Studies)

  - Task 3 (Q9): Make vs. Buy: Facilitate a discussion on the advantages of making the sieve versus buying it, from a business perspective.

  - Final Touches: Learners should complete their sieve construction and the electronic portfolio.

  - Submission: Establish clear submission guidelines for both the physical sieve (if required for display/demonstration) and the electronic portfolio.

3. Assessment and Monitoring

- Continuous Monitoring: Observe group dynamics, individual contributions, safety adherence, and skill development throughout the month.

- Formative Feedback: Provide regular feedback to groups on their progress, design choices, safety practices, and documentation.

- Summative Assessment: Use the provided marking scheme (which I cannot provide in detail here for confidentiality reasons) to evaluate:

  - The quality and completeness of the freehand sketch (Task 1, Q1).

  - The correctness of identified safety precautions and drawing lines (Task 1, Q2, Q3).

  - The understanding of marking/measuring importance (Task 1, Q4).

  - The completeness and quality of the digital documentation (Task 2, Q5).

  - The creation of the electronic portfolio and identification of the ICT tool (Task 2, Q6).

  - The accuracy of identified entrepreneurial qualities (Task 3, Q7).

  - The correct identification and classification of economic resources (Task 3, Q8).

  - The logical reasoning for the make-or-buy decision (Task 3, Q9).

### Apparatus, Items, Resources (General List for Learners)

A. Tools:

- Measuring & Marking:

  - Steel Rule / Tape Measure

  - Pencil (for marking wood/plastic)

  - Marker Pen (for marking metal/plastic)

  - Try Square (for marking right angles)

  - Divider/Compass (for marking circles if needed)

- Cutting:

  - Hacksaw (for metal or plastic)

  - Tenon Saw (for wood)

  - Scissors / Tin Snips (for mesh/fabric/thin plastic)

  - Craft Knife / Utility Knife (for thinner materials, with extreme caution)

- Joining/Fastening:

  - Hammer (for nails)

  - Screwdriver (Phillips/Flathead, matching screws)

  - Pliers (various types for bending wire, holding small parts)

  - Bench Vice / G-clamp (for holding workpieces securely)

- Shaping/Finishing:

  - Files (for smoothing edges of metal/plastic)

  - Sandpaper (for smoothing wood/plastic)

  - Wood Rasp (for rough shaping of wood)

- Safety Gear:

  - Safety Glasses / Goggles (essential for cutting, filing, hammering)

  - Gloves (for handling rough materials, preventing splinters)

  - Dust Mask (if sanding/creating dust)

  - Apron (to protect clothing)

B. Materials (Locally Available - Emphasize Reuse/Recycling):

- Frame/Body:

  - Wood offcuts (e.g., from old crates, furniture, timber scraps)

  - Thick plastic sheets / parts of old plastic containers (e.g., buckets, jerry cans)

  - Strong cardboard (for prototyping or lighter duty sieves)

  - Thin metal sheets (e.g., from old tins, scrap metal)

  - Thick wire (for a circular frame)

- Sieving Mesh/Surface:

  - Old metal sieves (repurposed)

  - Wire mesh/chicken wire (ensure small enough holes for flour)

  - Fabric mesh/netting (e.g., from old mosquito nets, strong curtain material – if fine enough)

  - Perforated plastic sheets

  - Old window screens

- Joining/Fastening:

  - Small nails

  - Wood screws / Self-tapping screws

  - Strong adhesive (e.g., wood glue, super glue, contact adhesive - suitable for materials used)

  - Staples (for fabric/mesh)

  - String / Twine / Cable ties

C. ICT Resources:

- Devices:

  - Smartphone / Digital Camera (for capturing photos/videos)

  - Computer / Laptop (for creating the electronic portfolio)

- Software/Platforms:

  - Photo Gallery/Viewer (built-in on devices)

  - Simple Video Editor (e.g., free mobile apps, built-in computer editors like Photos App on Windows, iMovie on Mac)

  - Presentation Software (e.g., Google Slides, Microsoft PowerPoint, LibreOffice Impress)

  - Cloud Storage (e.g., Google Drive, OneDrive, Dropbox - for sharing and backing up files)

  - Web Browser (for accessing cloud platforms)

### Procedures (General Steps for Making a Sieve - Learners' Approach)

- Understand the Need: Discuss what makes a good sieve for porridge flour (fine mesh, sturdy, easy to use).

- Design and Sketch (Task 1, Q1):

  - Brainstorm different sieve shapes (round, square) and materials.

  - Sketch a chosen design, showing the frame, handle (if any), and mesh area.

  - Add approximate dimensions (e.g., diameter/side length, depth).

  - Indicate materials to be used for each part.

- Material Sourcing:

  - Identify and collect suitable locally available materials.

  - Clean and prepare materials (e.g., remove old paint, straighten wire).

- Measuring and Marking (Task 1, Q4):

  - Accurately measure and mark the frame pieces and mesh material according to the sketch dimensions.

  - Double-check measurements before cutting.

- Cutting (Task 1, Q2):

  - Cut the frame pieces (wood, plastic, metal) using appropriate tools (tenon saw for wood, hacksaw for metal/plastic).

  - Cut the mesh material to size using scissors or tin snips.

  - Crucially, observe all safety precautions (safety glasses, stable workpiece, correct tool usage).

- Shaping/Finishing:

  - Smooth rough edges of cut pieces using sandpaper or files.

  - Drill holes if necessary for joining with screws.

- Assembly:

  - Assemble the frame using chosen joining methods (nails, screws, glue). Ensure it's sturdy.

  - Attach the mesh securely to the frame (staples, glue, wire, small nails). Ensure there are no gaps for flour to escape un-sieved.

- Final Checks:

  - Test the sieve for stability and effectiveness.

  - Ensure all sharp edges are smoothed and it's safe to handle.

- Documentation (Task 2, Q5):

  - Take photos or short videos at each major step of the process (sketching, marking, cutting, assembly, final product).

- Electronic Portfolio Creation (Task 2, Q6):

  - Gather all photos/videos.

  - Use an ICT tool (e.g., Google Slides, PowerPoint) to create a presentation.

  - Insert photos/videos chronologically, adding brief captions for each step.

  - Name the portfolio clearly (e.g., "Group Cheetah Sieve Project").

- Reflect on Business Aspects (Task 3, Q7, Q8, Q9):

  - Discuss the vendor's entrepreneurial qualities.

  - Identify economic resources used.

  - Consider why making is better than buying in this specific scenario.

Please note: The "Answers" section below is what a teacher would expect from the students, based on the question prompts in the learner paper.

### Expected Learner Answers (Non-Confidential Examples)

TASK 1: Pre-Technical Studies (10 marks)

- Make a freehand pictorial sketch of the sieve showing all important parts and dimensions. (Use locally available materials such as wood, metal, or plastic in your sketch.) (4 marks)

  - Expected Answer: A clear, hand-drawn 3D sketch of a sieve.

    - Parts: Clearly showing a frame (e.g., wooden square/circular frame), a mesh/netting part, and potentially a handle or supporting legs.

    - Dimensions: Indicating approximate sizes (e.g., 20cm x 20cm for square, 25cm diameter, 5cm depth).

    - Materials: Labeling or indicating the intended materials (e.g., "wooden frame," "wire mesh," "plastic handle").

    - Pictorial: Drawn in a way that shows three dimensions (e.g., isometric or oblique).

- Identify two safety precautions to observe when using cutting tools such as a hacksaw and a tenon saw. (2 marks)

  - a. Wear appropriate Personal Protective Equipment (PPE): Always wear safety glasses/goggles to protect eyes from flying sawdust, metal shavings, or splinters. Gloves can also be used for grip and to prevent cuts.

  - b. Secure the work-piece firmly: Ensure the material being cut (wood or metal) is held securely in a vice or clamped to a workbench. This prevents the material from moving, slipping, or causing injury during cutting, and allows for a more accurate cut.

  - c. Use the correct tool for the job: Use a hacksaw for metal/plastic and a tenon saw for wood. Using the wrong tool can damage the tool, the material, or cause injury.

  - d. Keep hands clear of the cutting path: Position hands away from the blade's path to avoid accidental cuts.

  - e. Maintain sharp tools: Dull tools require more force and are more likely to slip, leading to accidents.

- Name two types of lines that can be used to show hidden or internal features in your sieve drawing. (2 marks)

  - a. Hidden lines (or Dash lines): These are thin, dashed lines used to represent edges or features that are not directly visible from the current view of the object.

  - b. Centre lines (or Long-dash short-dash lines): While primarily used to indicate the center of circles, arcs, and cylindrical objects, they can also indirectly show the axis of symmetry for internal features if those features are cylindrical or symmetrical. (Though 'hidden lines' is the primary and most direct answer for hidden features).

- Explain two reasons why it is important to follow correct marking and measuring procedures when making the sieve. (2 marks)

  - a. To ensure accuracy and fit: Correct marking and measuring ensure that all parts of the sieve are cut to the precise dimensions required. This is crucial for the parts to fit together properly during assembly, resulting in a well-constructed and functional sieve.

  - b. To minimize material waste: Accurate marking and measuring reduce errors in cutting, thus preventing the need to discard incorrectly cut pieces. This conserves valuable materials, especially when using locally available or scarce resources.

  - c. To achieve desired functionality/quality: The sieve needs to be a specific size and have a properly secured mesh to effectively sift flour. Accurate dimensions ensure it performs its intended function efficiently and is of good quality.

  - d. To save time and effort: Reworking or re-cutting due to inaccurate measurements wastes time and effort. Following correct procedures from the start makes the construction process smoother and more efficient.

TASK 2: Computer Studies (10 marks)

- Take photos or short videos as you carry out each step of the project — from sketching, measuring, cutting, assembling, to final product. (4 marks)

Expected Outcome: Learner groups should have a collection of 5-10 clear photos or short video clips demonstrating their practical work at various stages of the sieve construction. Each stage (sketching, measuring, cutting, assembling, final product) should be clearly represented. The quality and relevance of the media would be assessed.

- Create an electronic portfolio named after your group (e.g., Group Simba) containing the videos/photos. Indicate one ICT tool you used in creating the portfolio. (6 marks)

a) Name the portfolio: E.g., "Group Simba Sieve Project Portfolio", "Group Eagles Pre-Tech Sieve". (The group's name should be included.)

b) ICT Tool used:

For capturing: Smartphone camera, Digital camera.

For creating portfolio (software/platform): Microsoft PowerPoint, Google Slides, Google Photos, any simple video editing software (e.g., iMovie, InShot), Google Drive (if linking shared folder).

For viewing: Web browser (e.g., Chrome, Firefox), Photo Viewer, Video Player.

(Learners only need to name ONE tool used for creation/assembly of the portfolio)

TASK 3: Business Studies (10 marks)

- From the scenario, state three qualities that describe the porridge vendor as an entrepreneur. (3 marks)

a. Initiative/Proactive: The vendor identifies a problem (porridge not well-sieved) and takes action to improve her business by seeking a solution (making a sieve).

b. Problem-solver: She recognizes a specific issue that affects her product's quality and customer satisfaction and seeks a practical solution to address it.

c. Resourceful: The vendor is looking for a practical solution, which implies she is thinking about how to get things done effectively to improve her product.

d. Quality-oriented/Customer-focused: By wanting "well-sieved flour," she demonstrates a focus on improving the quality of her product, which benefits her customers.

- State three economic resources the vendor used or could use in this project. Classify each as natural, human, or capital. (3 marks)

| Resource | Classification |
| --- | --- |
| 1. Wood / Metal / Plastic (for sieve) | Natural |
| 2. Baker's/Vendor's skills/labour | Human |
| 3. Tools (e.g., hacksaw, tenon saw) | Capital |
| 4. Time/Effort (to make the sieve) | Human |
| 5. Money (to buy materials/tools if not locally sourced) | Capital |

(Learners need to provide 3 distinct resources and classify them correctly.)

- Outline four reasons why it is better for the vendor to make the sieve rather than buy it. (4 marks)

a) Cost-effectiveness: Making the sieve using locally available and possibly repurposed materials (scrap wood, old mesh) is likely to be much cheaper than buying a ready-made one, especially if she has access to basic tools. This saves her business money.

b) Customization/Specific Fit: By making it herself, the vendor can design the sieve to her exact specifications – the desired size, mesh fineness, and material, ensuring it perfectly meets her unique needs for sieving porridge flour. A bought sieve might not be ideal.

c) Skill Development/Empowerment: Making the sieve herself allows her to gain practical skills in craftsmanship and problem-solving. This self-reliance can be empowering and useful for future small repairs or modifications.

d) Immediate Availability/No Waiting: If there are no ready-made sieves available for purchase nearby, or if she needs it urgently, making it herself ensures she can have it ready when needed, avoiding delays in her business operations.

e) Durability/Quality Control: By using chosen materials and overseeing the construction process, she can ensure the sieve is sturdy and made to last, potentially outperforming a cheaper, mass-produced alternative.

f) Resourcefulness/Sustainability: Using locally available or recycled materials promotes resourcefulness and sustainable practices, aligning with environmental consciousness.

THIS IS THE LAST PRINTED PAGE.