KENYA JUNIOR SCHOOL EDUCATION ASSESSMENT

GRADE 9: 912/2: PRETECHNICAL STUDIES

CODE: 008   YEAR: 2025

MARKING SCHEME

## CONFIDENTIAL

### PREPARATION GUIDELINES

General Instructions:

- Ensure the workshop/laboratory is clean, well-lit, and adequately ventilated.

- All tools must be in MEETING working condition, sharp where necessary, and readily accessible to students.

- Prepare sufficient quantities of all materials, ensuring they meet the specified dimensions.

- Emphasize safety throughout the project. Conduct a safety briefing before the practical work begins. Ensure appropriate PPE is available and used.

- The examiner must have a copy of the specific isometric sketch for Section 1 and the circuit diagram for Section 3 to verify student work.

### SECTION 1: TECHNICAL DRAWING (15 Marks)

Resources/Apparatus/Items for EACH Candidate:

- Drawing Board/Table: Stable and clean surface for drawing.

- T-square or Parallel Rule: For drawing horizontal lines.

- Set Squares (30/60 and 45 degree): For drawing vertical and angled lines.

- Pencils: HB and 2H recommended for construction and object lines.

- Eraser: High quality, non-smudging.

- Drawing Paper: A3 size recommended (or A4 if the object is small and simple), unlined.

- Measuring Rule (30cm): Metric scale.

- Dividers/Compasses: (Optional, depending on the complexity of the isometric object).

- The Isometric Object Sketch: This must be provided to the students on their question paper.

Examiner's Note on Isometric Object Sketch (Crucial Confidential Information): For the "porridge sieve" context, a simple, relevant shape would be a block with a cutout or a basic L-shaped bracket which can be later imagined as part of the sieve's frame. Example Isometric Sketch (for examiner's reference – dimensions are illustrative):

(Top View)

+-------+

/|      /|

/ |     / |

+--+----+  |  <-- Height (H) = 50mm

|  |    |  |

|  +----|--+  <-- Width (W) = 40mm

| /     | /

|/______|/

+-------+

<------->

Length (L) = 70mm

With a simple 'step' or 'cutout':

Imagine an L-shape block.

Dimensions:

Overall Length (L) = 70mm

Overall Width (W) = 40mm

Overall Height (H) = 50mm

Cutout/Step:

Cutout from top right corner: 30mm x 20mm (Length x Width)

Depth of cutout: 20mm (from top)

Procedure for Examiner/Teacher:

- Ensure each candidate has a clear printout of the isometric sketch with dimensions on their question paper.

- Provide drawing paper and necessary tools.

- Remind candidates of first-angle projection rules and the need for construction lines.

- Emphasize neatness, accuracy, and correct use of drawing instruments.

### SECTION 2: WORKSHOP TECHNOLOGY - WOODWORK (20 Marks)

Resources/Apparatus/Items for EACH Candidate/Group Access:

- Materials Provided (per student):

  - Timber Piece A: 1 piece of softwood, 150mm x 30mm x 20mm (ensure consistent dimensions).

  - Timber Piece B: 1 piece of softwood, 150mm x 30mm x 20mm (ensure consistent dimensions).

- Tools Required (per student/group access):

  - Marking Gauge: 1 per student/group.

  - Try Square: 1 per student/group.

  - Measuring Tape/Steel Rule: 1 per student/group.

  - Pencil: For marking out.

  - Tenon Saw: 1 per student/group (sharp and properly set).

  - Wood Chisel: 1 (e.g., 12mm or 18mm) per student/group (sharp).

  - Mallet/Hammer: 1 per student/group.

  - Vice: Workbench with a wood vice (at least one per two students).

  - Bench Hook: (Optional, but highly recommended for safety and accuracy during sawing) 1 per student/group.

  - Saw Horse/Work bench: Suitable work surface.

Examiner's Note on Half-Lap Joint Diagram (Crucial Confidential Information): The diagram provided to the students should clearly show the dimensions for the half-lap joint. For the given timber (30mm x 20mm):

- The cut should be 10mm deep (half of 20mm thickness).

- The length of the lap should be 30mm (equal to the width of the timber).

- The diagram should show two pieces joining to form a continuous surface.

Example Half-Lap Joint Diagram (for examiner's reference):

Piece A:           Piece B:

+----------+       +----------+

|          |       |          |

|          |       |          |

|          |       |          |

|          |       |          |

|          |       |          |

|          |       |          |

+----------+       +----------+

<-- 30mm -->        <-- 30mm -->

(Width)            (Width)

<----10mm----> (Cutout Depth = 1/2 thickness)

(from top)     (from bottom)

+----------+       +----------+

|          |       |          |

|  ______  |       |  ______  |

| |      | |       | |      | |

| |      | |       | |      | |

| |      | |       | |      | |

| |______| |       | |______| |

+----------+       +----------+

<-- 30mm -->        <-- 30mm -->

(Length of lap)    (Length of lap)

Procedure for Examiner/Teacher:

- Before the exam, ensure all timber pieces are precisely cut to the specified dimensions (150mm x 30mm x 20mm).

- Brief students on the proper use of all tools, especially the tenon saw, chisel, and mallet, emphasizing safety protocols (e.g., always cut away from the body, secure work in a vice).

- Ensure adequate workspace and tool access for all students.

- Monitor students during the marking out and cutting phases for accuracy and safety.

### SECTION 3: BASIC ELECTRICAL WIRING (15 Marks)

Resources/Apparatus/Items for EACH Candidate/Group Access:

- Materials Provided (per student/group):

  - Small Wooden or Plastic Baseboard: Approx. 100mm x 150mm. Pre-drill small pilot holes if necessary for easier mounting.

  - Battery Holder: 1 (for 2 AA or D cell batteries, depending on availability).

  - AA or D Cell Batteries: 2 (fresh, ensure compatibility with holder and bulbs).

  - SPST (Single Pole, Single Throw) Switch: 1.

  - Lamp Holders: 2 (small screw type or bayonet type, ensure compatible with bulbs).

  - Low Voltage Bulbs: 2 (e.g., 2.5V or 3V, compatible with batteries). Ensure they are working.

  - Insulated Connecting Wires: Approx. 1 meter, various colors (e.g., red and black) recommended for positive/negative identification.

  - Small Screws/Adhesive: For mounting components to the baseboard.

  - Small Terminal Block: (Optional, but MEETING for neat wiring) 1 or 2 sections.

- Tools Required (per student/group access):

  - Small Screwdriver: 1 (flat head and/or Phillips head, suitable for component terminals).

  - Wire Strippers/Side Cutters: 1 (for cutting and stripping insulation).

Examiner's Note on Parallel Circuit Diagram (Crucial Confidential Information): The diagram provided to the students must be a clear, standard parallel circuit diagram with a single battery source, a single switch, and two bulbs connected in parallel.

Example Parallel Circuit Diagram (for examiner's reference):

(A standard circuit diagram showing: Battery, Switch in series with the battery, and two bulbs connected in parallel across the output of the switch/battery. All connections should be clear)

Procedure for Examiner/Teacher:

- Ensure all electrical components are functional before the exam.

- Brief students on safe handling of electrical components, especially batteries and wires. Emphasize stripping insulation carefully to avoid short circuits.

- Monitor students during wiring to ensure correct connections and neatness.

- The final step requires students to demonstrate the working circuit to the examiner for functionality assessment.

### MARKING SCHEME (Teacher's Copy)

### SECTION 1: TECHNICAL DRAWING (15 Marks)

(a) Correct orthographic projection (first angle): 5 marks

| Rubric - First Angle Projection | Marks |
| --- | --- |
| EXCEEDING (5 marks): All three views (Front, End, Plan) are correctly projected in first-angle. No inversion or misalignment. | 5 |
| MEETING (3-4 marks): Two views correctly projected, or minor error in one projection (e.g., slight misalignment). | 3-4 |
| APPROACHING(2 marks): One view correctly projected, or significant misalignment/inversion in multiple views. | 2 |
| BELOW EX(0-1 mark): No recognizable orthographic projection or major conceptual errors. | 0-1 |

(b) Accuracy of lines and shapes: 3 marks

| Rubric - Line & Shape Accuracy | Marks |
| --- | --- |
| EXCEEDING (3 marks): All lines are straight, sharp, of appropriate thickness (object, hidden, center lines), and shapes are geometrically accurate. | 3 |
| MEETING (2 marks): Mostly accurate lines and shapes with minor inconsistencies or slight freehand appearance. | 2 |
| APPROACHING(1 mark): Noticeable inaccuracies in lines (wobbly, thick) and shapes, some features distorted. | 1 |
| BELOW EX(0 marks): Lines are messy, shapes are largely inaccurate or unrecognizable. | 0 |

(c) Inclusion and accuracy of 6 dimensions: 4 marks

| Rubric - Dimensions | Marks |
| --- | --- |
| EXCEEDING (4 marks): Six (6) key dimensions clearly shown, correctly placed (not on hidden lines), accurate values, and correctly formatted (dimension lines, extension lines, arrowheads). | 4 |
| MEETING (3 marks): 4-5 dimensions shown correctly, or 6 dimensions with minor errors in placement/format. | 3 |
| APPROACHING(2 marks): 2-3 dimensions shown correctly, or multiple errors in placement/format for 6 dimensions. | 2 |
| BELOW EX(0-1 mark): 0-1 dimension shown, or dimensions are largely incorrect/unreadable/poorly placed. | 0-1 |

(d) Neatness and clarity of drawing: 2 marks

| Rubric - Neatness & Clarity | Marks |
| --- | --- |
| EXCEEDING (2 marks): Drawing is exceptionally neat, clean, free of smudges, and easy to interpret. Construction lines are light. | 2 |
| MEETING (1 mark): Generally neat with minor smudges or slightly heavy construction lines, but still clear. | 1 |
| APPROACHING(0 marks): Messy, smudged, difficult to read, or excessively heavy construction lines. | 0 |

(e) Correct title block: 1 mark

| Rubric - Title Block | Marks |
| --- | --- |
| EXCEEDING (1 mark): Title block is present, correctly positioned, and contains all required information (Title "Object Drawing", Name/Assessment No., Scale "1:1"). | 1 |
| BELOW EX(0 marks): Title block missing, incomplete, or contains significant errors. | 0 |

### SECTION 2: WORKSHOP TECHNOLOGY - WOODWORK (20 Marks)

(a) Accuracy of marking out: 5 marks

| Rubric - Marking Out Accuracy | Marks |
| --- | --- |
| EXCEEDING (5 marks): All lines (shoulder lines, gauge lines) are precisely marked to specified dimensions on both pieces. Lines are sharp and clear. | 5 |
| MEETING (3-4 marks): Mostly accurate marking out, with minor deviations (e.g., +/- 1mm) or slightly unclear lines. | 3-4 |
| APPROACHING(2 marks): Noticeable inaccuracies in marking (e.g., +/- 2-3mm) or BELOW EXline clarity. | 2 |
| BELOW EX(0-1 mark): Significant errors in marking, making the joint impossible to construct accurately. | 0-1 |

(b) Accuracy of cuts (straightness, depth): 7 marks

| Rubric - Cut Accuracy | Marks |
| --- | --- |
| EXCEEDING (7 marks): All saw cuts (shoulder and cheek) are perfectly straight and plumb. Chisel cuts are clean and to the correct depth (10mm). No splintering. | 7 |
| MEETING (5-6 marks): Cuts are mostly straight and to depth with minor deviations or slight unevenness. Minimal splintering. | 5-6 |
| APPROACHING(3-4 marks): Cuts show noticeable deviations (e.g., not plumb, uneven depth). Some splintering. | 3-4 |
| BELOW EX(0-2 marks): Cuts are significantly off, very uneven, major splintering, or incorrect dimensions. | 0-2 |

(c) Quality of joint fit (flushness, tightness): 6 marks

| Rubric - Joint Fit | Marks |
| --- | --- |
| EXCEEDING (6 marks): Joint fits perfectly flush on all surfaces with no gaps. Requires gentle force to assemble, indicating a tight fit. | 6 |
| MEETING (4-5 marks): Joint is mostly flush with very minor gaps or slight unevenness. MEETING tightness. | 4-5 |
| APPROACHING(2-3 marks): Noticeable gaps (e.g., >1mm) or significant unevenness/step at the joint interface. Loose or very stiff fit. | 2-3 |
| BELOW EX(0-1 mark): Joint does not fit, or is extremely loose/gappy, indicating major errors. | 0-1 |

(d) Neatness and finish of component: 2 marks

| Rubric - Neatness & Finish | Marks |
| --- | --- |
| EXCEEDING (2 marks): The completed joint is clean, free of pencil marks, and any chisel marks are minimal and well-controlled. | 2 |
| MEETING (1 mark): Generally neat with minor pencil marks or slight evidence of tool marks, but acceptable. | 1 |
| BELOW EX(0 marks): Messy, excessive pencil marks, rough finish, or significant damage from tools. | 0 |

### SECTION 3: BASIC ELECTRICAL WIRING (15 Marks)

(a) Correct circuit assembly (parallel configuration): 7 marks

| Rubric - Circuit Assembly | Marks |
| --- | --- |
| EXCEEDING (7 marks): All components (battery holder, switch, 2 lamp holders) are correctly mounted and wired in a complete parallel circuit as per the diagram. | 7 |
| MEETING (5-6 marks): Most components are correctly wired in parallel, with only minor connection errors (e.g., one bulb slightly off connection, but still parallel). | 5-6 |
| APPROACHING(3-4 marks): Significant wiring errors (e.g., bulbs wired in series instead of parallel, or major part of the circuit incorrect). | 3-4 |
| BELOW EX(0-2 marks): Circuit is largely incorrect, incomplete, or does not resemble a parallel configuration. | 0-2 |

(b) Functionality (both bulbs light up): 4 marks

| Rubric - Functionality | Marks |
| --- | --- |
| EXCEEDING (4 marks): Both bulbs light up brightly and simultaneously when the switch is ON. | 4 |
| MEETING (3 marks): One bulb lights up brightly, the other is dim or flickers, or lights up slightly later. | 3 |
| APPROACHING(2 marks): Only one bulb lights up, or both are very dim/flickering constantly. | 2 |
| BELOW EX(0-1 mark): Neither bulb lights up, or circuit short-circuits. | 0-1 |

(c) Neatness of wiring: 2 marks

| Rubric - Wiring Neatness | Marks |
| --- | --- |
| EXCEEDING (2 marks): Wires are cut to appropriate lengths, routed neatly, and connections are tidy with minimal excess wire. | 2 |
| MEETING (1 mark): Wires are generally neat but may have some excess length or slight untidiness. | 1 |
| BELOW EX(0 marks): Wires are excessively long, tangled, messy, or connections are very untidy. | 0 |

(d) Safety in handling components and wires: 2 marks

| Rubric - Safety | Marks |
| --- | --- |
| EXCEEDING (2 marks): Demonstrates safe handling of tools and electrical components throughout the task (e.g., proper use of wire strippers, no short circuits, careful battery handling). | 2 |
| MEETING (1 mark): Mostly safe practices, with minor instances of carelessness (e.g., leaving bare wire exposed unnecessarily). | 1 |
| BELOW EX(0 marks): Demonstrates unsafe practices (e.g., using incorrect tools, shorting batteries, exposed wires). | 0 |

THIS IS THE LAST PRINTED PAGE.