| Candidate’s Name |  | Assessment Number |  |
| --- | --- | --- | --- |
| School Name |  | School Code |  |
| Candidate’s Signature |  | Date |  |

MONITORING JUNIOR SCHOOL LEARNER PROGRESS

GRADE 9: 912/2: PRETECHNICAL STUDIES.

PAPER 2. (Project)

CODE: 008   YEAR: 2025

DURATION: 1 Month

## INSTRUCTIONS TO CANDIDATES

- Read all instructions carefully before starting any task.

- Ensure you have all the necessary tools and materials as specified for each section.

- Observe all safety regulations and use appropriate Personal Protective Equipment (PPE) where necessary (e.g., safety glasses, gloves).

- All dimensions are in millimetres (mm) unless otherwise stated.

- Neatness, accuracy, and adherence to safety procedures will be assessed.

- Do NOT dismantle your work from Section 3 until instructed by the examiner.

### TASK

A porridge vendor who sells porridge on a construction site wants to make a sieve so that her porridge is made of well-sieved flour.

| Task | Question Numbers | Maximum Score | Learner’s Score |
| --- | --- | --- | --- |
| Task 1 | 1, 2, 3, 4 | 28 |  |
| Task 2 | 8, 9 | 6 |  |
| Task 3 | 5, 6, 7 | 6 |  |
| TOTAL |  | 40 Marks |  |

This paper consists of 5  printed pages. Candidates should check the question paper to ascertain that all the pages are printed as indicated and that no questions are missing.

© 2025 The Kenya Junior School Examination and Assessment Series.                         Turn over

### SECTION 1: TECHNICAL DRAWING (15 Marks)

Task:

Using the provided isometric view of the object below, draw the following orthographic views in first angle projection:

a) Front elevation (viewed in direction of arrow X)

b) End elevation (viewed from the right side)

c) Plan view (viewed from the top)

Requirements:

- Draw to a scale of 1:1 (Full Size).

- Add six (6) key dimensions to your drawing.

- Include a simple title block indicating "Object Drawing", your name/assessment number, and the scale used.

(Examiner: Provide a clear, simple isometric sketch of an object, e.g., an L-block or a step block, with clear dimensions. Example dimensions: Length 70mm, Width 40mm, Height 50mm, with one step/cutout.)

### SECTION 2: WORKSHOP TECHNOLOGY - WOODWORK (20 Marks)

Task: Construct a Half-Lap Joint as shown in the diagram below, using the two pieces of prepared timber provided.

Materials Provided (per student):

- Timber Piece A: 1 piece of softwood, 150mm×30mm×20mm

- Timber Piece B: 1 piece of softwood, 150mm×30mm×20mm

Tools Required (per student/group access):

- Marking gauge

- Try square

- Measuring tape/steel rule

- Pencil

- Tenon saw

- Wood chisel (e.g., 12mm or 18mm)

- Mallet/Hammer

- Vice

- Bench hook (optional but recommended)

Procedure:

- Mark out the joint dimensions accurately on both pieces of timber, referring to the diagram below. (Assume a standard half-lap joint where half the thickness is removed from each piece).

- Use a tenon saw to make the shoulder and cheek cuts.

- Use a chisel and mallet to remove the waste wood.

- Fit the two pieces together to form a flush joint.

(Examiner: Provide a simple line diagram of a standard half-lap joint, clearly showing the dimensions to be removed for fitting. Ensure the joint dimensions are appropriate for the provided timber size, e.g., 30mm width, 10mm deep cuts for a 20mm thick timber.)

### SECTION 3: BASIC ELECTRICAL WIRING (15 Marks)

Task: Assemble a Parallel Circuit on the provided baseboard to light two bulbs simultaneously, controlled by a single switch.

Materials Provided (per student/group):

- Small wooden or plastic baseboard (approx. 100mm×150mm)

- 1 x Battery holder (for 2 AA or D cell batteries)

- 2 x AA or D cell batteries (as per holder)

- 1 x SPST (Single Pole, Single Throw) switch

- 2 x Lamp holders (small screw type or bayonet type)

- 2 x Low voltage bulbs (e.g., 2.5V or 3V, compatible with batteries)

- Insulated connecting wires (approx. 1 meter, various colours recommended)

- Small terminal block (optional, but good for neat wiring)

Tools Required (per student/group access):

- Small screwdriver (flat head and/or Phillips head)

- Wire strippers/side cutters

Procedure:

- Mount the battery holder, switch, and two lamp holders onto the baseboard using small screws or adhesive, ensuring proper spacing for wiring.

- Cut and strip appropriate lengths of connecting wires.

- Wire the components to form a parallel circuit where both bulbs light up when the switch is ON.

- Ensure all connections are secure and insulation is intact.

- Once wired, connect the batteries and demonstrate the working circuit to the examiner.

(Examiner: Provide a clear circuit diagram of a parallel circuit with a battery, single switch, and two bulbs. Ensure all connections are clear and distinct.)

### MARKING CRITERIA:

SECTION 1: TECHNICAL DRAWING (15 Marks)

- Correct orthographic projection (first angle): 5 marks

- Accuracy of lines and shapes: 3 marks

- Inclusion and accuracy of 6 dimensions: 4 marks

- Neatness and clarity of drawing: 2 marks

- Correct title block: 1 mark

SECTION 2: WORKSHOP TECHNOLOGY - WOODWORK (20 Marks)

- Accuracy of marking out: 5 marks

- Accuracy of cuts (straightness, depth): 7 marks

- Quality of joint fit (flushness, tightness): 6 marks

- Neatness and finish of component: 2 marks

SECTION 3: BASIC ELECTRICAL WIRING (15 Marks)

- Correct circuit assembly (parallel configuration): 7 marks

- Functionality (both bulbs light up): 4 marks

- Neatness of wiring: 2 marks

- Safety in handling components and wires: 2 marks

THIS IS THE LAST PRINTED PAGE.